Федеральное государственное автономное образовательное

учреждение высшего образования

«Национальный исследовательский университет ИТМО»

Дисциплина: Проектирование вычислительных систем

# Лабораторная работа №4

Вариант 4

Выполнили: Барсуков Максим Андреевич,

группа P3415

Стригалев Никита Сергеевич,

группа P3412

Преподаватель:

Пинкевич Василий Юрьевич

2025 г.

Санкт-Петербург

# Содержание

Задание	3

Вариант: 4	4

Используемые контакты	5

Блок-схемы	6

Основная программа	6

Режим настройки мелодии	7

Описание функций кнопок	8

Раскладки	9

Музыкальная раскладка	9

Раскладка настройки частоты	9

Раскладка настройки времени	9

Исходный код	10

source.h	10

main.с	13

Вывод	36

# Задание

Разработать программу, которая использует интерфейс I2C для того, чтобы корректно считывать нажатие кнопок клавиатуры стенда SDK-1.1. Подсистема опроса клавиатуры должна удовлетворять следующим требованиям:

- реализуется защита от дребезга;

- нажатие кнопки фиксируется сразу после того, как было обнаружено, что кнопка нажата (с учетом защиты от дребезга), а не в момент отпускания кнопки; если необходимо, долгое нажатие может фиксироваться отдельно;

- кнопка, которая удерживается дольше, чем один цикл опроса, не считается повторно нажатой до тех пор, пока не будет отпущена (нет переповторов);

- распознается и корректно обрабатывается множественное нажатие (при нажатии более чем одной кнопки считается, что ни одна кнопка не нажата, если это не противоречит требованиям к программе);

- всем кнопкам назначаются коды от 1 до 12 (порядок на усмотрение исполнителей).

Программа должна иметь два режима работы, переключение между которыми производится по нажатию кнопки на боковой панели стенда:

- режим тестирования клавиатуры;

- прикладной режим.

Уведомление о смене режима выводится в UART.

- В режиме тестирования клавиатуры программа выводит в UART коды нажатых кнопок.

- В прикладном режиме программа обрабатывает нажатия кнопок и выполняет действия в соответствии с вариантом задания.

## Вариант: 4

Задания аналогичны вариантам лабораторной работы №3, за исключением того, что ввод символов должен выполняться не с клавиатуры через UART, а с помощью клавиатуры стенда. Выбор кнопок клавиатуры стенда, играющих роль кнопок клавиатуры компьютера должен выполняться по усмотрению исполнителей. В отчете необходимо привести описание функций кнопок в реализованной программе.

# Используемые контакты

Рисунок 1 – Используемые контакты

# Блок-схемы

## Основная программа

Рисунок 2 – Схема основной программы

## Режим настройки мелодии

Рисунок 3 – Схема режима настройки мелодии

# Описание функций кнопок

| Символ | Действие |
| --- | --- |
| «1» – «4» | Воспроизведение одной из стандартных мелодий. |
| «5» | Воспроизведение пользовательской мелодии. |
| «e» | Вход в меню настройки. |
| «x» | Остановить мелодию |
| «q» | Выйти без сохранения из настройки пользовательской мелодии. |
| «s» | Сохранение ноты. |
| «n» | Сохранение частоты. |
| Боковая кнопка | Сохранение мелодии. |

# Раскладки

## Музыкальная раскладка

| 1 | 2 | 3 |
| --- | --- | --- |
| 4 | 5 |  |
|  | e | x |

## Раскладка настройки частоты

| 1 | 2 | 3 |
| --- | --- | --- |
| 4 | 5 | 6 |
| 7 | 8 | 9 |
| 0 | n | q |

## Раскладка настройки времени

| 1 | 2 | 3 |
| --- | --- | --- |
| 4 | 5 | 6 |
| 7 | 8 | 9 |
| 0 | s | q |

# Исходный код

## source.h

#ifndef SOURCE_H_ #define SOURCE_H_ #include "stdint.h" #define C1 33 #define CS1 35 #define D1 37 #define DS1 39 #define E1 41 #define F1 44 #define FS1 46 #define G1 49 #define GS1 52 #define A1 55 #define AS1 58 #define B1 62 #define C2 65 #define CS2 69 #define D2 73 #define DS2 78 #define E2 82 #define F2 87 #define FS2 93 #define G2 98 #define GS2 104 #define A2 110 #define AS2 117 #define B2 123 #define C3 131 #define CS3 139 #define D3 147 #define DS3 156 #define E3 165 #define F3 175 #define FS3 185 #define G3 196 #define GS3 208 #define A3 220 #define AS3 233 #define B3 247 #define C4 262 #define CS4 277 #define D4 294 #define DS4 311 #define E4 330 #define F4 349 #define FS4 370 #define G4 392 #define GS4 415 #define A4 440 #define AS4 466 #define B4 494 #define C5 523 #define CS5 554 #define D5 587 #define DS5 622 #define E5 659 #define F5 698 #define FS5 740 #define G5 784 #define GS5 831 #define A5 880 #define AS5 932 #define B5 988 #define C6 1047 #define CS6 1109 #define D6 1175 #define DS6 1245 #define E6 1319 #define F6 1397 #define FS6 1480 #define G6 1568 #define GS6 1661 #define A6 1760 #define AS6 1865 #define B6 1976 #define C7 2093 #define CS7 2217 #define D7 2349 #define DS7 2489 #define E7 2637 #define F7 2794 #define FS7 2960 #define G7 3136 #define GS7 3322 #define A7 3520 #define AS7 3729 #define B7 3951 #define C8 4186 #define CS8 4435 #define D8 4699 #define DS8 4978 #define VOLUME_MAX 10 #define VOLUME_MIN 0 extern uint32_t stairway_melody[]; extern uint32_t stairway_delays[]; extern uint32_t deftones_melody[]; extern uint32_t ssshhhiiittt_delays[]; extern uint32_t duvet_melody[]; extern uint32_t duvet_delays[]; extern uint32_t radiohead_melody[]; extern uint32_t radiohead_delays[]; extern uint32_t user_melody[256]; extern uint32_t user_delays[256]; extern uint32_t user_melody_size; extern uint32_t start_melody[]; extern uint32_t start_delays[]; extern uint32_t stop_melody[]; extern uint32_t stop_delays[]; #endif

## main.с

/* USER CODE BEGIN PTD */ /* USER CODE BEGIN Header */ /** ****************************************************************************** * @file : main.c * @brief : Main program body ****************************************************************************** * @attention * * Copyright (c) 2022 STMicroelectronics. * All rights reserved. * * This software is licensed under terms that can be found in the LICENSE file * in the root directory of this software component. * If no LICENSE file comes with this software, it is provided AS-IS. * ****************************************************************************** */ /* USER CODE END Header */ /* Includes ------------------------------------------------------------------*/ #include "main.h" #include "i2c.h" #include "tim.h" #include "usart.h" #include "gpio.h" /* Private includes ----------------------------------------------------------*/ /* USER CODE BEGIN Includes */ #include "source.h" #include <stdio.h> #include <stdarg.h> #include <usart.h> #include <string.h> #include <ctype.h> #include <ctype.h> #include <stdbool.h> /* USER CODE END Includes */ /* Private typedef -----------------------------------------------------------*/ /* USER CODE END PTD */ /* Private define ------------------------------------------------------------*/ /* USER CODE BEGIN PD */ #define UART_TIMEOUT 10 #define BUF_SIZE 256 #define MAX_DIGITS 5 #define KB_I2C_ADDRESS (0xE2) #define KB_I2C_READ_ADDRESS ((KB_I2C_ADDRESS) ¦ 1) #define KB_I2C_WRITE_ADDRESS ((KB_I2C_ADDRESS) & ~1) #define KB_INPUT_REG (0x0) #define KB_OUTPUT_REG (0x1) #define KB_CONFIG_REG (0x3) #define KB_KEY_DEBOUNCE_TIME (200) /* USER CODE END PD */ /* Private macro -------------------------------------------------------------*/ /* USER CODE BEGIN PM */ /* USER CODE END PM */ /* Private variables ---------------------------------------------------------*/ /* USER CODE BEGIN PV */ uint32_t custom_track_data[256]; uint32_t custom_track_timing[256]; uint32_t custom_track_length = 0; uint32_t current_duration = 1; bool is_track_playing = true; uint32_t track_position = 0; uint32_t *current_track_notes; uint32_t *current_track_durations; uint32_t track_notes_count; uint32_t note_value = 0; uint32_t timing_value = 0; bool comma_detected = false; uint32_t stairway_melody[] = { A2, C3, E3, A4, B4, E3, C3, B4, C4, E3, C2, C4, FS3, D3, A2, FS3 }; uint32_t stairway_delays[] = { 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450 }; uint32_t ssshhhiiittt_melody[] = { E4, DS4, CS4, DS4, CS4, 0, B3, 0, CS4, 0, B3, CS4, DS4, 0, B3, E4, DS4, CS4, DS4, CS4, 0, B3, 0, CS4, 0, B3, CS4, DS4, 0, AS3 }; uint32_t ssshhhiiittt_delays[] = { 400, 400, 400, 400, 400, 70, 400, 70, 400, 70, 400, 300, 400, 100, 400, 400, 400, 400, 400, 400, 70, 400, 70, 400, 70, 400, 400, 400, 100, 400 }; uint32_t duvet_melody[] = { D5, CS5, B4, CS5, D5, 0, E5, FS5, B4, 0, D5, CS5, B4, CS5, D5, 0, E5, 0, FS5, E5, 0, FS5, E5, 0, E5, D5, CS5 }; uint32_t duvet_delays[] = { 150 * 3, 150* 3, 150* 3, 150* 3, 150* 3, 50, 150* 3, 150* 3, 250* 3, 50, 150* 3, 150* 3, 150* 3, 150* 3, 150* 3, 50, 150* 3, 100, 150* 3, 150* 3, 50, 150* 3, 150* 3, 100, 250* 3, 150* 3, 250 * 3 }; uint32_t radiohead_melody[] = { FS4, A3, D4, A3, FS4, A3, D4, A3, FS4, A3, D4, A3, G3, AS3, D4, E4, FS4, A3, D4, A3, FS4, A3, D4, A3, FS4, A3, D4, A3, G3, AS3, D4, E4 }; uint32_t radiohead_delays[] = { 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450 }; uint32_t start_melody[] = {C5, 0, E5, 0, G5}; uint32_t start_delays[] = {200, 50, 200, 50, 400}; uint32_t stop_melody[] = {0}; uint32_t stop_delays[] = {100}; uint32_t last_pressing_time = 0; int last_pressed_btn_index = -1; typedef enum { MODE_MUSIC, MODE_EDIT_FREQ, MODE_EDIT_TIME, MODE_TEST } SystemMode; SystemMode current_mode = MODE_MUSIC; SystemMode previous_mode = MODE_MUSIC; bool is_music_mode = true; bool is_settings_mode = false; bool is_test_keyboard_mode = false; bool last_btn_state = false; char music_layout[] = {'3', '2', '1', '6', '5', '4', '9', '8', '7', 'x', 'e', '0'}; char edit_freq_layout[] = {'3', '2', '1', '6', '5', '4', '9', '8', '7', 'q', 'n', '0'}; char edit_time_layout[] = {'3', '2', '1', '6', '5', '4', '9', '8', '7', 'q', 's', '0'}; char test_layout[] = {'3', '2', '1', '6', '5', '4', '9', '8', '7', '!', 't', '0'}; uint32_t temp_notes[256]; uint32_t temp_timings[256]; uint32_t temp_size = 0; uint32_t current_note_freq = 0; uint32_t current_note_time = 0; bool is_editing_freq = true; char freq_buffer[32] = {0}; char time_buffer[32] = {0}; /* USER CODE END PV */ /* Private function prototypes -----------------------------------------------*/ void SystemClock_Config(void); /* USER CODE BEGIN PFP */ /* USER CODE END PFP */ /* Private user code ---------------------------------------------------------*/ /* USER CODE BEGIN 0 */ static void set_green_led(bool on) { HAL_GPIO_WritePin(GPIOD, GPIO_PIN_13, on ? GPIO_PIN_SET : GPIO_PIN_RESET); } static void set_yellow_led(bool on) { HAL_GPIO_WritePin(GPIOD, GPIO_PIN_14, on ? GPIO_PIN_SET : GPIO_PIN_RESET); } static void set_red_led(bool on) { HAL_GPIO_WritePin(GPIOD, GPIO_PIN_15, on ? GPIO_PIN_SET : GPIO_PIN_RESET); } void blink_leds(int count) { for (int i = 0; i < count; i++) { set_green_led(true); set_yellow_led(true); set_red_led(true); HAL_Delay(100); set_green_led(false); set_yellow_led(false); set_red_led(false); if (i < count - 1) HAL_Delay(100); } } bool is_mode_btn_pressed(void) { return HAL_GPIO_ReadPin(GPIOC, GPIO_PIN_15) == GPIO_PIN_RESET; } int get_pressed_btn_index(void) { const uint32_t t = HAL_GetTick(); if (t - last_pressing_time < KB_KEY_DEBOUNCE_TIME) { return -1; } int index = -1; uint8_t reg_buffer = 0xFF; uint8_t tmp = 0; int detected_keys = 0; int row_values[4] = {0}; int col_values[3] = {0}; HAL_I2C_Mem_Write(&hi2c1, KB_I2C_WRITE_ADDRESS, KB_OUTPUT_REG, 1, &tmp, 1, KB_KEY_DEBOUNCE_TIME); for (int row = 0; row < 4; row++) { uint8_t config_value = ~((uint8_t)(1 << row)); HAL_I2C_Mem_Write(&hi2c1, KB_I2C_WRITE_ADDRESS, KB_CONFIG_REG, 1, &config_value, 1, KB_KEY_DEBOUNCE_TIME); HAL_Delay(2); HAL_I2C_Mem_Read(&hi2c1, KB_I2C_READ_ADDRESS, KB_INPUT_REG, 1, &reg_buffer, 1, KB_KEY_DEBOUNCE_TIME); uint8_t column_state = (reg_buffer >> 4) & 0x07; if ((column_state & 0x04) == 0) { if (detected_keys == 0) index = row * 3 + 0; detected_keys++; row_values[row]++; col_values[0]++; } if ((column_state & 0x02) == 0) { if (detected_keys == 0) index = row * 3 + 1; detected_keys++; row_values[row]++; col_values[1]++; } if ((column_state & 0x01) == 0) { if (detected_keys == 0) index = row * 3 + 2; detected_keys++; row_values[row]++; col_values[2]++; } } if (detected_keys > 2) { return -1; } int rows_pressed = 0; int cols_pressed = 0; for (int i = 0; i < 4; i++) { if (row_values[i] > 0) rows_pressed++; } for (int i = 0; i < 3; i++) { if (col_values[i] > 0) cols_pressed++; } if (rows_pressed != 1 ¦¦ cols_pressed != 1) { return -1; } if (detected_keys == 1 && index != -1) { if (index == last_pressed_btn_index) { return -1; } last_pressed_btn_index = index; last_pressing_time = t; return index; } if (detected_keys == 0) { last_pressed_btn_index = -1; } return -1; } char key2char(int key_index) { if (key_index < 0 ¦¦ key_index >= 12) { return '!'; } switch (current_mode) { case MODE_EDIT_FREQ: return edit_freq_layout[key_index]; case MODE_EDIT_TIME: return edit_time_layout[key_index]; case MODE_TEST: return test_layout[key_index]; case MODE_MUSIC: default: return music_layout[key_index]; } } void enter_edit_mode(void) { previous_mode = current_mode; current_mode = MODE_EDIT_FREQ; is_editing_freq = true; current_note_freq = 0; current_note_time = 0; set_green_led(true); set_yellow_led(false); set_red_led(false); char msg[] = "\r\n=== РЕЖИМ РЕДАКТИРОВАНИЯ ===\r\n" "Редактирование частоты ноты\r\n" "Используйте цифры 0-9 для ввода\r\n" "'n' - перейти к времени\r\n" "'q' - выйти без сохранения\r\n" "=============================\r\n"; HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); } void exit_edit_mode(bool save) { current_mode = MODE_MUSIC; set_green_led(false); set_yellow_led(is_track_playing); set_red_led(false); char msg[] = "\r\n=== ВОЗВРАТ В МУЗЫКАЛЬНЫЙ РЕЖИМ ===\r\n"; HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); } void process_edit_input(char input_char) { if (input_char == 'q') { char msg[32]; sprintf(msg, "\r\nВыход. Нот: %lu\r\n", temp_size); HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); current_mode = MODE_MUSIC; set_green_led(false); set_yellow_led(is_track_playing); set_red_led(false); return; } if (input_char == 'n') { if (is_editing_freq) { is_editing_freq = false; current_mode = MODE_EDIT_TIME; char msg[64]; sprintf(msg, "\r\nВремя для %lu Гц: ", current_note_freq); HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); } return; } if (input_char == 's') { if (current_mode == MODE_EDIT_TIME) { if (temp_size < 256) { temp_notes[temp_size] = current_note_freq; temp_timings[temp_size] = current_note_time; temp_size++; char msg[64]; if (current_note_freq == 0) { sprintf(msg, "\r\n[%lu] Пауза: %lu мс\r\n", temp_size, current_note_time); } else { sprintf(msg, "\r\n[%lu] Нота: %lu Гц, %lu мс\r\n", temp_size, current_note_freq, current_note_time); } HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); blink_leds(1); current_note_freq = 0; current_note_time = 0; is_editing_freq = true; current_mode = MODE_EDIT_FREQ; HAL_UART_Transmit(&huart6, (uint8_t*)"\r\nСлед. нота: ", 13, 100); } else { HAL_UART_Transmit(&huart6, (uint8_t*)"\r\nЛимит!\r\n", 9, 100); } } return; } if (input_char >= '0' && input_char <= '9') { uint8_t digit = input_char - '0'; if (current_mode == MODE_EDIT_FREQ) { if (current_note_freq > 100000) { HAL_UART_Transmit(&huart6, (uint8_t*)"\r\nСлишком много!\r\n", 17, 100); return; } current_note_freq = current_note_freq * 10 + digit; char msg[32]; sprintf(msg, "\r\nЧастота: %lu\r\n", current_note_freq); HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); } else if (current_mode == MODE_EDIT_TIME) { if (current_note_time > 10000) { HAL_UART_Transmit(&huart6, (uint8_t*)"\r\nСлишком много!\r\n", 17, 100); return; } current_note_time = current_note_time * 10 + digit; char msg[32]; sprintf(msg, "\r\nВремя: %lu\r\n", current_note_time); HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); } return; } char msg[32]; sprintf(msg, "\r\nНеизв: %c\r\n", input_char); HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); } void process_settings_input(char input_char) { char response[128]; if (input_char == 'q') { is_music_mode = true; is_settings_mode = false; set_green_led(false); set_yellow_led(false); set_red_led(false); char msg[] = "\r\n=== РЕЖИМ МУЗЫКИ ===\r\n"; HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); return; } if (input_char == 's') { if (temp_size > 0) { for (uint32_t i = 0; i < temp_size; i++) { custom_track_data[i] = temp_notes[i]; custom_track_timing[i] = temp_timings[i]; } custom_track_length = temp_size; char msg[64]; sprintf(msg, "\r\nСохранено нот: %lu\r\n=== РЕЖИМ МУЗЫКИ ===\r\n", temp_size); HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); blink_leds(2); is_music_mode = true; is_settings_mode = false; set_green_led(false); set_yellow_led(false); set_red_led(false); } else { HAL_UART_Transmit(&huart6, (uint8_t*)"\r\nНет нот для сохранения!\r\n", 26, 100); } return; } if (input_char == '?') { char help[] = "\r\n--- СПРАВКА ---\r\n" "Формат: частота,длительность\r\n" "Пример: 440,500 - нота Ля 440Гц на 500мс\r\n" "Пример: 0,200 - пауза 200мс\r\n" "'s' - сохранить, 'q' - выйти\r\n" "----------------\r\n"; HAL_UART_Transmit(&huart6, (uint8_t*)help, strlen(help), 100); return; } if (input_char == ',') { if (comma_detected) { HAL_UART_Transmit(&huart6, (uint8_t*)"\r\nОшибка: две запятые!\r\n", 23, 100); note_value = 0; timing_value = 0; comma_detected = false; } else { comma_detected = true; HAL_UART_Transmit(&huart6, (uint8_t*)",", 1, 10); } return; } if (input_char == '\r') { if (temp_size < 256) { temp_notes[temp_size] = note_value; temp_timings[temp_size] = timing_value; temp_size++; char msg[64]; if (note_value == 0) { sprintf(msg, "\r\n[%lu] Пауза: %lu мс\r\n", temp_size, timing_value); } else { sprintf(msg, "\r\n[%lu] Нота: %lu Гц, %lu мс\r\n", temp_size, note_value, timing_value); } HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); set_yellow_led(true); HAL_Delay(50); set_yellow_led(false); note_value = 0; timing_value = 0; comma_detected = false; } else { HAL_UART_Transmit(&huart6, (uint8_t*)"\r\nЛимит нот (256)!\r\n", 19, 100); } return; } if (!isdigit((unsigned char)input_char)) { HAL_UART_Transmit(&huart6, (uint8_t*)"\r\nТолько цифры 0-9!\r\n", 20, 100); note_value = 0; timing_value = 0; comma_detected = false; return; } if (temp_size >= 256) { HAL_UART_Transmit(&huart6, (uint8_t*)"\r\nДостигнут лимит!\r\n", 18, 100); return; } if (comma_detected) { timing_value = timing_value * 10 + (input_char - '0'); } else { note_value = note_value * 10 + (input_char - '0'); } HAL_UART_Transmit(&huart6, (uint8_t*)&input_char, 1, 10); } void handle_key_press(int btn_index) { char received_char = key2char(btn_index); if (current_mode == MODE_TEST) { char msg[8]; sprintf(msg, "[%c]\r\n", received_char); HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); return; } if (current_mode == MODE_EDIT_FREQ ¦¦ current_mode == MODE_EDIT_TIME) { process_edit_input(received_char); return; } char response[128]; switch (received_char) { case '1': { sprintf(response, "\r\n1 - Led Zeppelin - Stairway to Heaven\r\n"); HAL_UART_Transmit(&huart6, (uint8_t*)response, strlen(response), 100); stop_playback(); start_track_playback(stairway_melody, stairway_delays, sizeof(stairway_melody) / sizeof(uint32_t)); blink_leds(1); break; } case '2': { sprintf(response, "\r\n2 - Ssshhhiiittt! - Серф Панк\r\n"); HAL_UART_Transmit(&huart6, (uint8_t*)response, strlen(response), 100); stop_playback(); start_track_playback(ssshhhiiittt_melody, ssshhhiiittt_delays, sizeof(ssshhhiiittt_melody) / sizeof(uint32_t)); blink_leds(2); break; } case '3': { sprintf(response, "\r\n3 - Boa - Duvet\r\n"); HAL_UART_Transmit(&huart6, (uint8_t*)response, strlen(response), 100); stop_playback(); start_track_playback(duvet_melody, duvet_delays, sizeof(duvet_melody) / sizeof(uint32_t)); blink_leds(3); break; } case '4': { sprintf(response, "\r\n4 - Radiohead - No Surprises\r\n"); HAL_UART_Transmit(&huart6, (uint8_t*)response, strlen(response), 100); stop_playback(); start_track_playback(radiohead_melody, radiohead_delays, sizeof(radiohead_melody) / sizeof(uint32_t)); blink_leds(4); break; } case '5': { if (custom_track_length > 0) { sprintf(response, "\r\n5 - Пользовательская мелодия (%lu нот)\r\n", custom_track_length); HAL_UART_Transmit(&huart6, (uint8_t*)response, strlen(response), 100); stop_playback(); start_track_playback(custom_track_data, custom_track_timing, custom_track_length); blink_leds(5); } else { sprintf(response, "\r\n!!! Нет пользовательской мелодии\r\n" "Нажмите 'e' для создания\r\n"); HAL_UART_Transmit(&huart6, (uint8_t*)response, strlen(response), 100); } break; } case 'x': { sprintf(response, "\r\n>>> Остановка\r\n"); HAL_UART_Transmit(&huart6, (uint8_t*)response, strlen(response), 100); stop_playback(); set_green_led(false); set_yellow_led(false); set_red_led(false); break; } case '?': { char menu[] = "\r\n=== МЕНЮ ===\r\n" "1 - Stairway to Heaven\r\n" "2 - Deftones\r\n" "3 - Boa Duvet\r\n" "4 - Radiohead\r\n" "5 - Пользовательская\r\n" "x - Стоп\r\n" "e - Редактирование\r\n" "? - Справка\r\n" "=============\r\n"; HAL_UART_Transmit(&huart6, (uint8_t*)menu, strlen(menu), 100); break; } case 'e': { if (current_mode != MODE_EDIT_FREQ && current_mode != MODE_EDIT_TIME) { enter_edit_mode(); } else { char msg[64]; if (is_editing_freq) { sprintf(msg, "\r\nРежим: ввод частоты. Текущая: %lu Гц\r\n", current_note_freq); } else { sprintf(msg, "\r\nРежим: ввод времени. Текущее: %lu мс\r\n", current_note_time); } HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); } break; } default: { if (received_char != '!') { sprintf(response, "\r\n??? Неизвестная команда: %c\r\n", received_char); HAL_UART_Transmit(&huart6, (uint8_t*)response, strlen(response), 100); } break; } } } void save_custom_melody(void) { if (temp_size > 0) { for (uint32_t i = 0; i < temp_size; i++) { custom_track_data[i] = temp_notes[i]; custom_track_timing[i] = temp_timings[i]; } custom_track_length = temp_size; HAL_UART_Transmit(&huart6, (uint8_t*)"\r\n[SAVED]\r\n", 10, 10); set_green_led(true); HAL_Delay(50); set_green_led(false); } else { HAL_UART_Transmit(&huart6, (uint8_t*)"\r\n[NO NOTES]\r\n", 14, 10); } } void show_custom_melody(void) { if (temp_size > 0) { char msg[64]; sprintf(msg, "\r\nМелодия: %lu нот\r\n", temp_size); HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); HAL_Delay(10); uint32_t notes_to_show = temp_size; if (notes_to_show > 5) notes_to_show = 5; for (uint32_t i = 0; i < notes_to_show; i++) { if (temp_notes[i] == 0) { sprintf(msg, "[%lu] Пауза: %lu мс\r\n", i+1, temp_timings[i]); } else { sprintf(msg, "[%lu] Нота: %lu Гц, %lu мс\r\n", i+1, temp_notes[i], temp_timings[i]); } HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); HAL_Delay(5); } if (temp_size > 5) { HAL_UART_Transmit(&huart6, (uint8_t*)"... и еще нот\r\n", 14, 100); } } else { HAL_UART_Transmit(&huart6, (uint8_t*)"\r\nНет мелодии!\r\n", 15, 100); } HAL_Delay(10); } struct RingBuffer { char data[BUF_SIZE]; uint16_t head; uint16_t tail; uint16_t count; }; typedef struct RingBuffer RingBuffer; static void buf_init(RingBuffer *buf) { buf->head = 0; buf->tail = 0; buf->count = 0; memset(buf->data, 0, BUF_SIZE); } static bool buf_push(RingBuffer *buf, char c) { if (buf->count >= BUF_SIZE) { return false; } buf->data[buf->head] = c; buf->head = (buf->head + 1) % BUF_SIZE; buf->count++; return true; } static bool buf_pop(RingBuffer *buf, char *c) { if (buf->count == 0) { return false; } *c = buf->data[buf->tail]; buf->tail = (buf->tail + 1) % BUF_SIZE; buf->count--; return true; } static RingBuffer rx_buffer; static RingBuffer ringBufferRx; static RingBuffer ringBufferTx; static char input_char[2] = {"\0"}; static uint8_t uart_rx_char; struct SystemState { bool interrupts_active; }; static struct SystemState system_state; bool transmit_busy = false; void activate_interrupts(struct SystemState *state) { HAL_NVIC_EnableIRQ(USART6_IRQn); state->interrupts_active = true; HAL_UART_Receive_IT(&huart6, (uint8_t*)&uart_rx_char, 1); } void deactivate_interrupts(struct SystemState *state) { HAL_NVIC_DisableIRQ(USART6_IRQn); state->interrupts_active = false; } void send_uart(char *buffer, size_t length) { HAL_UART_Transmit(&huart6, (uint8_t*)buffer, length, 100); } void send_uart_line(char *buffer, size_t length) { send_uart(buffer, length); send_uart("\r\n", 2); } void receive_uart_data(const struct SystemState *state) { if (state->interrupts_active) { return; } HAL_StatusTypeDef result = HAL_UART_Receive(&huart6, (uint8_t*)input_char, 1, 0); if (result == HAL_OK) { buf_push(&ringBufferRx, input_char); send_uart_data(state, input_char, 1); } } void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart_instance) { if (huart_instance->Instance == USART6) { buf_push(&rx_buffer, uart_rx_char); HAL_UART_Receive_IT(&huart6, (uint8_t*)&uart_rx_char, 1); } } void HAL_UART_TxCpltCallback(UART_HandleTypeDef *huart_instance) { char output_buffer[1024]; if (buf_pop(&ringBufferTx, output_buffer)) { HAL_UART_Transmit_IT(&huart6, (uint8_t*)output_buffer, strlen(output_buffer)); } else { transmit_busy = false; } } void start_track_playback(uint32_t *notes_array, uint32_t *timings_array, uint32_t array_length) { track_position = 0; current_track_notes = notes_array; current_track_durations = timings_array; track_notes_count = array_length; is_track_playing = true; current_duration = 1; HAL_TIM_PWM_Start(&htim1, TIM_CHANNEL_1); HAL_TIM_Base_Start_IT(&htim6); set_yellow_led(true); } void stop_playback(void) { is_track_playing = false; __HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, 0); track_position = 0; current_duration = 1; set_yellow_led(false); } void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim) { if (htim->Instance == TIM6) { if (!is_track_playing) { return; } if (current_duration > 0) current_duration--; if (current_duration == 0) { if (track_position >= track_notes_count) { stop_playback(); return; } uint32_t note_freq = current_track_notes[track_position]; uint32_t note_duration = current_track_durations[track_position]; if (note_freq == 0) { __HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, 0); current_duration = note_duration; } else { current_duration = note_duration; uint32_t arr_value = (2000000 / note_freq) - 1; __HAL_TIM_SET_AUTORELOAD(&htim1, arr_value); __HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, arr_value / 2); } track_position++; if (track_position >= track_notes_count) { stop_playback(); } } } } /* USER CODE END 0 */ /** * @brief The application entry point. * @retval int */ int main(void) { /* USER CODE BEGIN 1 */ /* USER CODE END 1 */ /* MCU Configuration--------------------------------------------------------*/ /* Reset of all peripherals, Initializes the Flash interface and the Systick. */ HAL_Init(); /* USER CODE BEGIN Init */ /* USER CODE END Init */ /* Configure the system clock */ SystemClock_Config(); /* USER CODE BEGIN SysInit */ /* USER CODE END SysInit */ /* Initialize all configured peripherals */ MX_GPIO_Init(); MX_USART6_UART_Init(); MX_TIM1_Init(); MX_TIM6_Init(); MX_I2C1_Init(); /* USER CODE BEGIN 2 */ HAL_TIM_PWM_Start(&htim1, TIM_CHANNEL_1); HAL_TIM_Base_Start_IT(&htim6); bool program_initialized = true; current_track_notes = start_melody; current_track_durations = start_delays; track_notes_count = sizeof(start_melody) / sizeof(uint32_t); char welcome_message[] = "\r\n" "Музыкальная шкатулка с клавиатурой I2C\r\n" "Музыкальный режим:\r\n" " 1-4 - стандартные мелодии\r\n" " 5 - пользовательская мелодия\r\n" " x - остановка воспроизведения\r\n" " e - режим редактирования\r\n" " ? - справка\r\n" "\r\n" "Режим редактирования:\r\n" " 0-9 - ввод цифр\r\n" " n - следующий параметр\r\n" " s - сохранить ноту\r\n" " q - отмена/выход\r\n" "\r\n" "Кнопка PC15 - тестовый режим\r\n" "========================================\r\n"; send_uart_line(welcome_message, strlen(welcome_message)); start_track_playback(start_melody, start_delays, sizeof(start_melody) / sizeof(uint32_t)); blink_leds(3); uint8_t init_config = 0xFF; HAL_I2C_Mem_Write(&hi2c1, KB_I2C_WRITE_ADDRESS, KB_CONFIG_REG, 1, &init_config, 1, 100); set_green_led(false); set_yellow_led(false); set_red_led(false); uint32_t last_mode_change = 0; bool mode_btn_pressed = false; buf_init(&rx_buffer); send_uart_line(welcome_message, strlen(welcome_message)); program_initialized = false; /* USER CODE END 2 */ /* Infinite loop */ /* USER CODE BEGIN WHILE */ while (1) { static uint32_t last_cleanup = 0; if (HAL_GetTick() - last_cleanup > 1000) { __HAL_UART_CLEAR_FLAG(&huart6, UART_FLAG_TC); __HAL_UART_CLEAR_FLAG(&huart6, UART_FLAG_TXE); last_cleanup = HAL_GetTick(); } bool current_btn_state = is_mode_btn_pressed(); if (current_btn_state && !mode_btn_pressed) { mode_btn_pressed = true; last_mode_change = HAL_GetTick(); } else if (!current_btn_state && mode_btn_pressed) { mode_btn_pressed = false; uint32_t press_duration = HAL_GetTick() - last_mode_change; if (press_duration > 50 && press_duration < 500) { if (current_mode == MODE_TEST) { current_mode = MODE_MUSIC; char msg[] = "\r\n=== ТЕСТОВЫЙ РЕЖИМ ВЫКЛЮЧЕН ===\r\n"; HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); set_red_led(false); } else { previous_mode = current_mode; current_mode = MODE_TEST; char msg[] = "\r\n=== ТЕСТОВЫЙ РЕЖИМ ВКЛЮЧЕН ===\r\n" "Нажимайте клавиши для теста\r\n" "==============================\r\n"; HAL_UART_Transmit(&huart6, (uint8_t*)msg, strlen(msg), 100); set_red_led(true); } } else if (press_duration >= 1000) { save_custom_melody(); } } int btn_index = get_pressed_btn_index(); if (btn_index != -1) { handle_key_press(btn_index); } if (current_mode == MODE_MUSIC && is_track_playing) { static uint32_t last_blink = 0; if (HAL_GetTick() - last_blink > 200) { set_yellow_led(!HAL_GPIO_ReadPin(GPIOD, GPIO_PIN_14)); last_blink = HAL_GetTick(); } } /* USER CODE END WHILE */ /* USER CODE BEGIN 3 */ } } /* USER CODE END 3 */ /** * @brief System Clock Configuration * @retval None */ void SystemClock_Config(void) { RCC_OscInitTypeDef RCC_OscInitStruct = {0}; RCC_ClkInitTypeDef RCC_ClkInitStruct = {0}; /** Configure the main internal regulator output voltage */ __HAL_RCC_PWR_CLK_ENABLE(); __HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1); /** Initializes the RCC Oscillators according to the specified parameters * in the RCC_OscInitTypeDef structure. */ RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSE; RCC_OscInitStruct.HSEState = RCC_HSE_ON; RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON; RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSE; RCC_OscInitStruct.PLL.PLLM = 15; RCC_OscInitStruct.PLL.PLLN = 216; RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2; RCC_OscInitStruct.PLL.PLLQ = 4; if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK) { Error_Handler(); } /** Activate the Over-Drive mode */ if (HAL_PWREx_EnableOverDrive() != HAL_OK) { Error_Handler(); } /** Initializes the CPU, AHB and APB buses clocks */ RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK¦RCC_CLOCKTYPE_SYSCLK ¦RCC_CLOCKTYPE_PCLK1¦RCC_CLOCKTYPE_PCLK2; RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK; RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1; RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV4; RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2; if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_5) != HAL_OK) { Error_Handler(); } } /* USER CODE BEGIN 4 */ /* USER CODE END 4 */ /** * @brief This function is executed in case of error occurrence. * @retval None */ void Error_Handler(void) { /* USER CODE BEGIN Error_Handler_Debug */ /* User can add his own implementation to report the HAL error return state */ __disable_irq(); while (1) { set_green_led(true); set_yellow_led(true); set_red_led(true); HAL_Delay(200); set_green_led(false); set_yellow_led(false); set_red_led(false); HAL_Delay(200); } /* USER CODE END Error_Handler_Debug */ } #ifdef USE_FULL_ASSERT /** * @brief Reports the name of the source file and the source line number * where the assert_param error has occurred. * @param file: pointer to the source file name * @param line: assert_param error line source number * @retval None */ void assert_failed(uint8_t *file, uint32_t line) { /* USER CODE BEGIN 6 */ /* User can add his own implementation to report the file name and line number, ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */ /* USER CODE END 6 */ } #endif /* USE_FULL_ASSERT */

# Вывод

В ходе выполнения лабораторной работы функционал музыкальной шкатулки был перенесен на клавиатуру. Это позволило задействовать интерфейс UART исключительно для информационного вывода. Через интерфейс I2C реализована система сменных раскладок, обеспечивающая работу в различных режимах и возможность ввода пользовательских мелодий.