Федеральное государственное автономное образовательное

учреждение высшего образования

«Национальный исследовательский университет ИТМО»

Дисциплина: Проектирование вычислительных систем

# Лабораторная работа №3

Вариант 4

Выполнили: Барсуков Максим Андреевич,

группа P3415

Стригалев Никита Сергеевич,

группа P3412

Преподаватель:

Пинкевич Василий Юрьевич

2025 г.

Санкт-Петербург

# Содержание

Задание	3

Вариант: 4	4

Блок-схемы	5

Основная программа	5

Режим настройки мелодии	6

Исходный код	7

source.h	7

main.h	9

main.с	10

Вывод	23

# Задание

Разработать программу, которая использует таймеры для управления яркостью светодиодов и излучателем звука (по прерыванию или с использованием аппаратных каналов). Блокирующее ожидание (функция HAL_Delay()) в программе использоваться не должно.

Стенд должен поддерживать связь с компьютером по UART и выполнять указанные действия в качестве реакции на нажатие кнопок на клавиатуре компьютера. В данной лабораторной работе каждая нажатая кнопка (символ, отправленный с компьютера на стенд) обрабатываются отдельно, ожидание ввода полной строки не требуется.

Для работы с UART на стенде можно использован один из двух вариантов драйвера (по прерыванию и по опросу) на выбор исполнителя. Поддержка двух вариантов не требуется.

Частота синхросигнала процессорного ядра и сигнала ШИМ для управления яркостью светодиодов (если используется) должны соответствовать указанным в варианте задания.

## Вариант: 4

Реализовать «музыкальную шкатулку» с мелодиями, которые состоят из последовательности звуков определенной частоты и длительности, а также пауз. Шкатулка должна иметь четыре стандартные мелодии и одну пользовательскую, которую можно настроить.

Действия стенда при получении символов от компьютера:

| Символ | Действие |
| --- | --- |
| «1» – «4» | Воспроизведение одной из стандартных мелодий. |
| «5» | Воспроизведение пользовательской мелодии. |
| «Enter» | Вход в меню настройки. |
| По усмотрению исполнителей | Ввод параметров пользовательской мелодии: частота (нота, октава), длительность, конец мелодии и т. п. |

После ввода каждого символа в UART должно выводиться сообщение о том, какой режим активирован, или текущие настройки, вводимые в меню.

Частота процессорного ядра – 180 МГц.

# Блок-схемы

## Основная программа

Рисунок 1 – Схема основной программы

## Режим настройки мелодии

Рисунок 2 – Схема режима настройки мелодии

# Исходный код

## source.h

#ifndef SOURCE_H_ #define SOURCE_H_ #include "stdint.h" #define C1 33 #define CS1 35 #define D1 37 #define DS1 39 #define E1 41 #define F1 44 #define FS1 46 #define G1 49 #define GS1 52 #define A1 55 #define AS1 58 #define B1 62 #define C2 65 #define CS2 69 #define D2 73 #define DS2 78 #define E2 82 #define F2 87 #define FS2 93 #define G2 98 #define GS2 104 #define A2 110 #define AS2 117 #define B2 123 #define C3 131 #define CS3 139 #define D3 147 #define DS3 156 #define E3 165 #define F3 175 #define FS3 185 #define G3 196 #define GS3 208 #define A3 220 #define AS3 233 #define B3 247 #define C4 262 #define CS4 277 #define D4 294 #define DS4 311 #define E4 330 #define F4 349 #define FS4 370 #define G4 392 #define GS4 415 #define A4 440 #define AS4 466 #define B4 494 #define C5 523 #define CS5 554 #define D5 587 #define DS5 622 #define E5 659 #define F5 698 #define FS5 740 #define G5 784 #define GS5 831 #define A5 880 #define AS5 932 #define B5 988 #define C6 1047 #define CS6 1109 #define D6 1175 #define DS6 1245 #define E6 1319 #define F6 1397 #define FS6 1480 #define G6 1568 #define GS6 1661 #define A6 1760 #define AS6 1865 #define B6 1976 #define C7 2093 #define CS7 2217 #define D7 2349 #define DS7 2489 #define E7 2637 #define F7 2794 #define FS7 2960 #define G7 3136 #define GS7 3322 #define A7 3520 #define AS7 3729 #define B7 3951 #define C8 4186 #define CS8 4435 #define D8 4699 #define DS8 4978 #define VOLUME_MAX 10 #define VOLUME_MIN 0 extern uint32_t stairway_melody[]; extern uint32_t stairway_delays[]; extern uint32_t deftones_melody[]; extern uint32_t ssshhhiiittt_delays[]; extern uint32_t duvet_melody[]; extern uint32_t duvet_delays[]; extern uint32_t radiohead_melody[]; extern uint32_t radiohead_delays[]; extern uint32_t user_melody[256]; extern uint32_t user_delays[256]; extern uint32_t user_melody_size; // Стартовая мелодия (короткая) extern uint32_t start_melody[]; extern uint32_t start_delays[]; // Стоп мелодия (тишина) extern uint32_t stop_melody[]; extern uint32_t stop_delays[]; #endif

## main.h

/* USER CODE BEGIN Header */ /** ****************************************************************************** * @file : main.h * @brief : Header for main.c file. * This file contains the common defines of the application. ****************************************************************************** * @attention * * Copyright (c) 2025 STMicroelectronics. * All rights reserved. * * This software is licensed under terms that can be found in the LICENSE file * in the root directory of this software component. * If no LICENSE file comes with this software, it is provided AS-IS. * ****************************************************************************** */ /* USER CODE END Header */ /* Define to prevent recursive inclusion -------------------------------------*/ #ifndef __MAIN_H #define __MAIN_H #ifdef __cplusplus extern "C" { #endif /* Includes ------------------------------------------------------------------*/ #include "stm32f4xx_hal.h" /* Private includes ----------------------------------------------------------*/ /* USER CODE BEGIN Includes */ #include "source.h" #include <stdbool.h> /* USER CODE END Includes */ /* Exported types ------------------------------------------------------------*/ /* USER CODE BEGIN ET */ /* USER CODE END ET */ /* Exported constants --------------------------------------------------------*/ /* USER CODE BEGIN EC */ /* USER CODE END EC */ /* Exported macro ------------------------------------------------------------*/ /* USER CODE BEGIN EM */ /* USER CODE END EM */ /* Exported functions prototypes ---------------------------------------------*/ void Error_Handler(void); /* USER CODE BEGIN EFP */ extern uint32_t custom_track_data[256]; extern uint32_t custom_track_timing[256]; extern uint32_t custom_track_length; extern uint32_t current_duration; extern bool is_track_playing; extern uint32_t track_position; extern uint32_t *current_track_notes; extern uint32_t *current_track_durations; extern uint32_t track_notes_count; void start_track_playback(uint32_t *notes_array, uint32_t *timings_array, uint32_t array_length); void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *timer_instance); /* USER CODE END EFP */ /* Private defines -----------------------------------------------------------*/ /* USER CODE BEGIN Private defines */ /* USER CODE END Private defines */ #ifdef __cplusplus } #endif #endif /* __MAIN_H */

## main.с

/* USER CODE BEGIN Header */ /** ****************************************************************************** * @file : main.c * @brief : Main program body ****************************************************************************** * @attention * * Copyright (c) 2022 STMicroelectronics. * All rights reserved. * * This software is licensed under terms that can be found in the LICENSE file * in the root directory of this software component. * If no LICENSE file comes with this software, it is provided AS-IS. * ****************************************************************************** */ /* USER CODE END Header */ /* Includes ------------------------------------------------------------------*/ #include "main.h" #include "tim.h" #include "usart.h" #include "gpio.h" /* Private includes ----------------------------------------------------------*/ /* USER CODE BEGIN Includes */ #include "source.h" #include <stdio.h> #include <stdarg.h> #include <usart.h> #include <string.h> #include <ctype.h> #include <ctype.h> #include <stdbool.h> /* USER CODE END Includes */ /* Private typedef -----------------------------------------------------------*/ /* USER CODE BEGIN PTD */ /* USER CODE END PTD */ /* Private define ------------------------------------------------------------*/ /* USER CODE BEGIN PD */ #define UART_TIMEOUT 10 #define BUF_SIZE 256 #define MAX_DIGITS 5 /* USER CODE END PD */ /* Private macro -------------------------------------------------------------*/ /* USER CODE BEGIN PM */ /* USER CODE END PM */ /* Private variables ---------------------------------------------------------*/ /* USER CODE BEGIN PV */ uint32_t custom_track_data[256]; uint32_t custom_track_timing[256]; uint32_t custom_track_length = 0; uint32_t current_duration = 1; bool is_track_playing = true; uint32_t track_position = 0; uint32_t *current_track_notes; uint32_t *current_track_durations; uint32_t track_notes_count; uint32_t stairway_melody[] = { A2, C3, E3, A4, B4, E3, C3, B4, C4, E3, C2, C4, FS3, D3, A2, FS3 }; uint32_t stairway_delays[] = { 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450 }; uint32_t ssshhhiiittt_melody[] = { E4, DS4, CS4, DS4, CS4, 0, B3, 0, CS4, 0, B3, CS4, DS4, 0, B3, E4, DS4, CS4, DS4, CS4, 0, B3, 0, CS4, 0, B3, CS4, DS4, 0, AS3 }; uint32_t ssshhhiiittt_delays[] = { 400, 400, 400, 400, 400, 70, 400, 70, 400, 70, 400, 300, 400, 100, 400, 400, 400, 400, 400, 400, 70, 400, 70, 400, 70, 400, 400, 400, 100, 400 }; uint32_t duvet_melody[] = { D5, CS5, B4, CS5, D5, 0, E5, FS5, B4, 0, D5, CS5, B4, CS5, D5, 0, E5, 0, FS5, E5, 0, FS5, E5, 0, E5, D5, CS5 }; uint32_t duvet_delays[] = { 150 * 3, 150* 3, 150* 3, 150* 3, 150* 3, 50, 150* 3, 150* 3, 250* 3, 50, 150* 3, 150* 3, 150* 3, 150* 3, 150* 3, 50, 150* 3, 100, 150* 3, 150* 3, 50, 150* 3, 150* 3, 100, 250* 3, 150* 3, 250 * 3 }; uint32_t radiohead_melody[] = { FS4, A3, D4, A3, FS4, A3, D4, A3, FS4, A3, D4, A3, G3, AS3, D4, E4, FS4, A3, D4, A3, FS4, A3, D4, A3, FS4, A3, D4, A3, G3, AS3, D4, E4 }; uint32_t radiohead_delays[] = { 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450, 450 }; uint32_t user_melody[256] = {0}; uint32_t user_delays[256] = {0}; uint32_t user_melody_size = 0; uint32_t start_melody[] = {C5, 0, E5, 0, G5}; uint32_t start_delays[] = {200, 50, 200, 50, 400}; uint32_t stop_melody[] = {0}; uint32_t stop_delays[] = {100}; /* USER CODE END PV */ /* Private function prototypes -----------------------------------------------*/ void SystemClock_Config(void); /* USER CODE BEGIN PFP */ /* USER CODE END PFP */ /* Private user code ---------------------------------------------------------*/ /* USER CODE BEGIN 0 */ struct RingBuffer { char data[BUF_SIZE]; uint16_t head; uint16_t tail; uint16_t count; }; typedef struct RingBuffer RingBuffer; static void buf_init(RingBuffer *buf) { buf->head = 0; buf->tail = 0; buf->count = 0; memset(buf->data, 0, BUF_SIZE); } static bool buf_push(RingBuffer *buf, char c) { if (buf->count >= BUF_SIZE) { return false; } buf->data[buf->head] = c; buf->head = (buf->head + 1) % BUF_SIZE; buf->count++; return true; } static bool buf_pop(RingBuffer *buf, char *c) { if (buf->count == 0) { return false; } *c = buf->data[buf->tail]; buf->tail = (buf->tail + 1) % BUF_SIZE; buf->count--; return true; } static RingBuffer rx_buffer; static RingBuffer ringBufferRx; static RingBuffer ringBufferTx; static char input_char[2] = {"\0"}; static uint8_t uart_rx_char; struct SystemState { bool interrupts_active; }; static struct SystemState system_state; bool transmit_busy = false; void activate_interrupts(struct SystemState *state) { HAL_NVIC_EnableIRQ(USART6_IRQn); state->interrupts_active = true; HAL_UART_Receive_IT(&huart6, (uint8_t*)&uart_rx_char, 1); } void deactivate_interrupts(struct SystemState *state) { HAL_NVIC_DisableIRQ(USART6_IRQn); state->interrupts_active = false; } void send_uart(char *buffer, size_t length) { HAL_UART_Transmit(&huart6, (uint8_t*)buffer, length, 100); } void send_uart_line(char *buffer, size_t length) { send_uart(buffer, length); send_uart("\r\n", 2); } void receive_uart_data(const struct SystemState *state) { if (state->interrupts_active) { return; } HAL_StatusTypeDef result = HAL_UART_Receive(&huart6, (uint8_t*)input_char, 1, 0); if (result == HAL_OK) { buf_push(&ringBufferRx, input_char); send_uart_data(state, input_char, 1); } } void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart_instance) { if (huart_instance->Instance == USART6) { buf_push(&rx_buffer, uart_rx_char); send_uart(&uart_rx_char, 1); HAL_UART_Receive_IT(&huart6, (uint8_t*)&uart_rx_char, 1); } } void HAL_UART_TxCpltCallback(UART_HandleTypeDef *huart_instance) { char output_buffer[1024]; if (buf_pop(&ringBufferTx, output_buffer)) { HAL_UART_Transmit_IT(&huart6, (uint8_t*)output_buffer, strlen(output_buffer)); } else { transmit_busy = false; } } void start_track_playback(uint32_t *notes_array, uint32_t *timings_array, uint32_t array_length) { track_position = 0; current_track_notes = notes_array; current_track_durations = timings_array; track_notes_count = array_length; is_track_playing = true; current_duration = 1; } void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim) { if (htim->Instance == TIM6) { if (!is_track_playing) { return; } if (current_duration > 0) current_duration--; if (current_duration == 0) { if (track_position >= track_notes_count) { is_track_playing = false; __HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, 0); return; } uint32_t note_freq = current_track_notes[track_position]; uint32_t note_duration = current_track_durations[track_position]; if (note_freq == 0) { // Пауза __HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, 0); current_duration = note_duration; } else { // Нота current_duration = note_duration; // Расчет для 180 МГц: TIM1 Prescaler = 89 -> тактовая = 2 МГц // ARR = 2000000 / freq - 1 uint32_t arr_value = (2000000 / note_freq) - 1; __HAL_TIM_SET_AUTORELOAD(&htim1, arr_value); __HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, arr_value / 2); } track_position++; if (track_position >= track_notes_count) { is_track_playing = false; __HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, 0); } } } } /* USER CODE END 0 */ /** * @brief The application entry point. * @retval int */ int main(void) { /* USER CODE BEGIN 1 */ /* USER CODE END 1 */ /* MCU Configuration--------------------------------------------------------*/ /* Reset of all peripherals, Initializes the Flash interface and the Systick. */ HAL_Init(); /* USER CODE BEGIN Init */ /* USER CODE END Init */ /* Configure the system clock */ SystemClock_Config(); /* USER CODE BEGIN SysInit */ /* USER CODE END SysInit */ /* Initialize all configured peripherals */ MX_GPIO_Init(); MX_USART6_UART_Init(); MX_TIM1_Init(); MX_TIM6_Init(); /* USER CODE BEGIN 2 */ HAL_TIM_PWM_Start(&htim1, TIM_CHANNEL_1); HAL_TIM_Base_Start_IT(&htim6); bool program_initialized = true; current_track_notes = start_melody; current_track_durations = start_delays; track_notes_count = sizeof(start_melody) / sizeof(uint32_t); char welcome_message[] = "Добро пожаловать в музыкальную шкатулку!"; char menu_options[] = "\r\nДоступные мелодии:\r\n" "\t1. - Led Zeppelin: Stairway to Heaven\r\n" "\t2. - Deftones: Be Quiet and Drive\r\n" "\t3. - Boa: Duvet\r\n" "\t4. - Radiohead: No Surprises\r\n" "\t5. - Пользовательская мелодия\r\n" "\tEnter - Настройка пользовательской мелодии\r\n" "\tx - Остановить воспроизведение"; char playback_stopped_msg[] = " - Воспроизведение остановлено!"; char invalid_selection_msg[] = " - Неверный выбор!"; char stairway_playing_msg[] = " - Играет: \"Stairway to Heaven\""; char deftones_playing_msg[] = " - Играет: \"Серф Панк\""; char duvet_playing_msg[] = " - Играет: \"Duvet\""; char radiohead_playing_msg[] = " - Играет: \"No Surprises\""; char custom_track_playing_msg[] = " - Играет пользовательская мелодия"; char no_custom_track_msg[] = " - Пользовательская мелодия не создана!"; char settings_menu[] = "Режим настройки пользовательской мелодии!\r\n" "Формат ввода: ЧАСТОТА,ДЛИТЕЛЬНОСТЬ\\r\\n\r\n" "'q' - выход без сохранения\r\n" "'s' - сохранить и выйти"; char note_added_msg[] = " - Нота добавлена!"; char format_error_msg[] = " - Ошибка формата! Повторите ввод."; char track_too_long_msg[] = "\r\nМелодия слишком длинная!"; char track_saved_msg[] = " - Мелодия сохранена!"; char settings_exit_msg[] = " - Выход из режима настройки"; buf_init(&rx_buffer); activate_interrupts(&system_state); send_uart_line(welcome_message, strlen(welcome_message)); program_initialized = false; //buf_init(&ringBufferRx); //buf_init(&ringBufferTx); /* USER CODE END 2 */ /* Infinite loop */ /* USER CODE BEGIN WHILE */ while (1) { if (program_initialized){ send_uart_line(welcome_message, strlen(welcome_message)); program_initialized = false; } char received_char; // Обрабатываем только по одному символу за раз if (buf_pop(&rx_buffer, &received_char)) { // ДЕБАГ: выводим что получили // char debug_msg[32]; // sprintf(debug_msg, "[DEBUG: 0x%02X '%c']", received_char, received_char); // send_uart_line(debug_msg, strlen(debug_msg)); switch (received_char) { case '1': { send_uart_line(stairway_playing_msg, strlen(stairway_playing_msg)); start_track_playback(stairway_melody, stairway_delays, sizeof(stairway_melody) / sizeof(uint32_t)); break; } case '2': { send_uart_line(deftones_playing_msg, strlen(deftones_playing_msg)); start_track_playback(ssshhhiiittt_melody, ssshhhiiittt_delays, sizeof(ssshhhiiittt_melody) / sizeof(uint32_t)); break; } case '3': { send_uart_line(duvet_playing_msg, strlen(duvet_playing_msg)); start_track_playback(duvet_melody, duvet_delays, sizeof(duvet_melody) / sizeof(uint32_t)); break; } case '4': { send_uart_line(radiohead_playing_msg, strlen(radiohead_playing_msg)); start_track_playback(radiohead_melody, radiohead_delays, sizeof(radiohead_melody) / sizeof(uint32_t)); break; } case '5': { if (custom_track_length != 0){ send_uart_line(custom_track_playing_msg, strlen(custom_track_playing_msg)); start_track_playback(custom_track_data, custom_track_timing, custom_track_length); } else { send_uart_line(no_custom_track_msg, strlen(no_custom_track_msg)); } break; } case '?': { send_uart_line(menu_options, strlen(menu_options)); break; } case 'x': { start_track_playback(stop_melody, stop_delays, sizeof(stop_melody) / sizeof(uint32_t)); send_uart_line(playback_stopped_msg, strlen(playback_stopped_msg)); break; } case 'e': { send_uart_line(settings_menu, strlen(settings_menu)); uint32_t temp_notes[256]; uint32_t temp_timings[256]; uint32_t current_size = 0; uint32_t note_value = 0; uint32_t timing_value = 0; bool comma_detected = false; bool settings_mode = true; while (settings_mode) { char input_char; if (buf_pop(&rx_buffer, &input_char)) { // send_uart(&input_char, 1); if (input_char == 'q') { send_uart_line(settings_exit_msg, strlen(settings_exit_msg)); settings_mode = false; } else if (input_char == 's') { if (current_size > 0) { send_uart_line(track_saved_msg, strlen(track_saved_msg)); for (uint32_t i = 0; i < current_size; i++) { custom_track_data[i] = temp_notes[i]; custom_track_timing[i] = temp_timings[i]; } custom_track_length = current_size; } else { send_uart_line(" - Мелодия пустая! Нечего сохранять.", 37); } settings_mode = false; } else if (input_char == ',') { if (comma_detected) { send_uart_line(format_error_msg, strlen(format_error_msg)); note_value = 0; timing_value = 0; comma_detected = false; } else { comma_detected = true; } } else if (input_char == '\r') { if (current_size < 256) { temp_notes[current_size] = note_value; temp_timings[current_size] = timing_value; current_size++; char note_msg[64]; if (note_value == 0) { snprintf(note_msg, sizeof(note_msg), " - Пауза %d мс добавлена! (нота %d)", timing_value, current_size); } else { snprintf(note_msg, sizeof(note_msg), " - Нота %d Гц, %d мс добавлена! (нота %d)", note_value, timing_value, current_size); } send_uart_line(note_msg, strlen(note_msg)); note_value = 0; timing_value = 0; comma_detected = false; } else { send_uart_line(track_too_long_msg, strlen(track_too_long_msg)); } } else if (!isdigit((unsigned char)input_char)) { send_uart_line(format_error_msg, strlen(format_error_msg)); note_value = 0; timing_value = 0; comma_detected = false; } else if (current_size >= 256) { send_uart_line(track_too_long_msg, strlen(track_too_long_msg)); } else { if (comma_detected) { timing_value = timing_value * 10 + (input_char - '0'); } else { note_value = note_value * 10 + (input_char - '0'); } } } HAL_Delay(10); } break; } case '\r': // Enter case '\n': // Новая строка break; default: { if (received_char >= 32 && received_char <= 126) { send_uart_line(invalid_selection_msg, strlen(invalid_selection_msg)); } break; } } } // Небольшая задержка для стабильности HAL_Delay(50); } /* USER CODE END WHILE */ /* USER CODE BEGIN 3 */ } /* USER CODE END 3 */ /** * @brief System Clock Configuration * @retval None */ void SystemClock_Config(void) { RCC_OscInitTypeDef RCC_OscInitStruct = {0}; RCC_ClkInitTypeDef RCC_ClkInitStruct = {0}; /** Configure the main internal regulator output voltage */ __HAL_RCC_PWR_CLK_ENABLE(); __HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1); /** Initializes the RCC Oscillators according to the specified parameters * in the RCC_OscInitTypeDef structure. */ RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSE; RCC_OscInitStruct.HSEState = RCC_HSE_ON; RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON; RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSE; RCC_OscInitStruct.PLL.PLLM = 15; RCC_OscInitStruct.PLL.PLLN = 216; RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2; RCC_OscInitStruct.PLL.PLLQ = 4; if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK) { Error_Handler(); } /** Activate the Over-Drive mode */ if (HAL_PWREx_EnableOverDrive() != HAL_OK) { Error_Handler(); } /** Initializes the CPU, AHB and APB buses clocks */ RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK¦RCC_CLOCKTYPE_SYSCLK ¦RCC_CLOCKTYPE_PCLK1¦RCC_CLOCKTYPE_PCLK2; RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK; RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1; RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV4; RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2; if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_5) != HAL_OK) { Error_Handler(); } } /* USER CODE BEGIN 4 */ /* USER CODE END 4 */ /** * @brief This function is executed in case of error occurrence. * @retval None */ void Error_Handler(void) { /* USER CODE BEGIN Error_Handler_Debug */ /* User can add his own implementation to report the HAL error return state */ __disable_irq(); while (1) { } /* USER CODE END Error_Handler_Debug */ } #ifdef USE_FULL_ASSERT /** * @brief Reports the name of the source file and the source line number * where the assert_param error has occurred. * @param file: pointer to the source file name * @param line: assert_param error line source number * @retval None */ void assert_failed(uint8_t *file, uint32_t line) { /* USER CODE BEGIN 6 */ /* User can add his own implementation to report the file name and line number, ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */ /* USER CODE END 6 */ } #endif /* USE_FULL_ASSERT */

# Вывод

В ходе выполнения лабораторной работы была реализована простая музыкальная шкатулка, способная воспроизводить четыре предопределенные мелодии, а также мелодию, заданную пользователем. Для этого был добавлен специальный режим настройки пользовательской мелодии. Для обеспечения взаимодействия с пользователем через последовательный интерфейс была интегрирована часть кода из предыдущей лабораторной работы, реализующая работу с UART — включая прием и обработку входных данных.